Benchmark

Pattern Series MAG1125-MAG1128

Random • Reversible

Width: 36” Hyacinth Leaves Wallcovering

Roll Length: 8 linear yards

Packaging: Double Rolls

Backing: 425gsm Non-Woven

Performance:

Fire Rating – Passes ASTM E84 – Class A

Flame Spread:  Pass

Smoke Developed: Pass

Notable Facts:

- This is a handmade wallcovering and is crafted with natural leaves and fibers which may result in an unevenness of finish or irregularities in the leaves.

- The uniqueness of each leaf creates an inherent variation of design, dimension and adds the natural beauty of the product.

- Color Variation and Design Variation are to be expected.

- Paneling effect, is a natural characteristic of handmade wallcoverings and adds to the uniqueness in addition to its natural beauty.

- NOT recommended for high traffic areas

- AVOID high moisture/humid areas.  Moisture can cause mold or mildew and cause the edges to curl away from the backing and/or delamination.

- DO NOT BACKROLL the material, as it will relax once pasted.  Back rolling may cause unnecessary stress to the Natural Surface.

- DO NOT ALLOW paste or excessive water to stain the front of the wallcoverings.  Both paste, and excessive water will change the color of the natural surface.

- Subtle shading differences between strips tend to soften over time as the wallcovering begins to oxidize.  DO NOT REVERSE.

- CARE-this is NOT a surface washable wallcovering.  To clean, wipe with a dry, lint-free cloth or using a soft brush attachment of a vacuum cleaner.

Design Copyright © York										     	                 7/2018