Colt

Pattern Series MAG1130-MAG1133

8” Repeat, Non- Reversible, Drop Match

Width: 36” Horse Hair, Metallic Yarn, Abaca Wallcovering

Roll Length: 8 linear yards

Packaging: Double Rolls

Backing: 300 gsm Non-Woven

Meets CA01350 Indoor Air Quality

Performance:

Fire Rating – Passes ASTM E84 – Class A

Flame Spread – Pass

Smoke Developed – Pass

Notable Facts:

- This is a natural wallcovering made with horse hair, metallic yarn and abaca which is hand cut and laid on the backing.

- Color Variation and Design Variation are to be expected.

- Paneling effect, is a natural characteristic of handmade wallcoverings and adds to the unique natural beauty of the product.

- NOT recommended for high traffic areas

- AVOID high moisture/humid areas.  Moisture can cause mold or mildew and cause the edges to curl away from the backing and/or delamination.

- DO NOT BACKROLL the material, as it will relax once pasted.  Back rolling may cause unnecessary stress to the Natural Surface.

- DO NOT ALLOW paste or excessive water to stain the front of the wallcoverings.  Both paste, and excessive water will change the color of the natural surface.

- Subtle shading differences between strips tend to soften over time as the wallcovering begins to oxidize.  DO NOT REVERSE.

- CARE-this is NOT a surface washable wallcovering.  To clean, wipe with a dry, lint-free cloth or using a soft brush attachment of a vacuum cleaner.

- There may be extended lead times due to it being a handcrafted item.

Design Copyright © York   				    				     	                                              7/2019