Tudor

Pattern Series MAG1105-MAG1110

7”-8” Straight • Non-Reverse

Width: 36” Wood Veneer and Foil Wallcovering

Roll Length: 8 linear yards

Packaging: Double Rolls

Backing:  300gsm Non-Woven

Meets CA01350 Indoor Air Quality

Performance:

Fire Rating – Passes ASTM E84 – Class A

Flame Spread:  Pass

Smoke Developed: Pass

Notable Facts:

- This is a natural wallcovering made with authentic wood veneer which is hand cut and laid on the backing.

- Color Variation and Design Variation are to be expected.

- Paneling effect, is a natural characteristic of handmade wallcoverings and adds to the unique natural beauty of the product.

- NOT recommended for high traffic areas

- AVOID high moisture/humid areas.  Moisture can cause mold or mildew and cause the edges to curl away from the backing and/or delamination.

- DO NOT BACKROLL the material, as it will relax once pasted.  Back rolling may cause unnecessary stress to the Natural Surface.

- DO NOT ALLOW paste or excessive water to stain the front of the wallcoverings.  Both paste, and excessive water will change the color of the natural surface.

- Subtle shading differences between strips tend to soften over time as the wallcovering begins to oxidize.  DO NOT REVERSE.

- CARE-this is NOT a surface washable wallcovering.  To clean, wipe with a dry, lint-free cloth or using a soft brush attachment of a vacuum cleaner.

Design Copyright © York										     	                 7/2018